| CONNECTING ROD BOLT |  |
| --- | --- |
| GRADE | 8.8/10.9/12.9 |
| STANDARD | ISO 898-1 |
| STRUCTURE | FINE PITCH ROLLED THREAD, PRECISION SHANK WITH CONTROLLED UNDER-HEAD RADIUS. |

| WHEEL BOLT |  |
| --- | --- |
| GRADE | 10.9 |
| STANDARD | DIN 74361 |
| STRUCTURE | HEXAGONAL HEAD WITH CONICAL (60°) OR SPHERICAL SEAT FOR WHEEL CENTERING. METRIC THREADED END WITH A PLAIN SHANK, DESIGNED TO CARRY SHEAR LOAD AND PROVIDE SECURE CLAMPING. |

| STUD |  |
| --- | --- |
| GRADE | 10.9 |
| STANDARD | DIN 938 / DIN 939 |
| STRUCTURE | HEADLESS FASTENER WITH EXTERNAL THREADS ON BOTH ENDS AND A PLAIN CENTRAL SHANK. |

| FLANGE BOLT |  |
| --- | --- |
| GRADE | 8.8/10.9 |
| STANDARD | DIN 6921 |
| STRUCTURE | HEXAGONAL HEAD BOLT WITH AN INTEGRAL CIRCULAR FLANGE UNDER THE HEAD AND METRIC EXTERNAL THREAD. |

| DOWEL PIN |  |
| --- | --- |
| GRADE | HARDENED |
| STANDARD | ISO 8734 (DIN 6325) |
| STRUCTURE | CYLINDRICAL SOLID PIN WITH CHAMFERED ENDS. |

| TRACK SHOE BOLT |  |
| --- | --- |
| GRADE | 10.9 / 12.9 |
| STANDARD | DIN 864 |
| STRUCTURE | LOW-PROFILE OVAL HEAD BOLT WITH NIBS UNDER THE HEAD, METRIC EXTERNAL THREAD, DESIGNED TO LOCK INTO THE TRACK SHOE AND PREVENT ROTATION. |

| TOOTH BOLT |  |
| --- | --- |
| GRADE | 10.9 |
| STANDARD | DIN 608 |
| STRUCTURE | PLOW HEAD (FLAT COUNTERSUNK) BOLT WITH SQUARE NECK BENEATH THE HEAD TO PREVENT ROTATION, METRIC EXTERNAL THREAD. |

| SHOULDER BOLT |  |
| --- | --- |
| GRADE | 10.9 / 12.9 |
| STANDARD | ISO 7379 (DIN 7379) |
| STRUCTURE | BOLT WITH A PRECISION GROUND CYLINDRICAL SHOULDER (UNTHREADED SHANK) UNDER THE HEAD AND A SHORT METRIC THREADED END. |

| AXLE |  |
| --- | --- |
| GRADE | 10.9 / 12.9 |
| STANDARD | AS PER DRAWING |
| STRUCTURE | SOLID OR HOLLOW CYLINDRICAL SHAFT WITH PRECISION-MACHINED ENDS FOR BEARING AND WHEEL MOUNTING; MAY INCLUDE SPLINES, KEYWAYS, OR THREADED PORTIONS DEPENDING ON DESIGN. |

| TORX SCREW |  |
| --- | --- |
| GRADE | 8.8/10.9/12.9 |
| STANDARD | ISO 14579 / DIN 7985 |
| STRUCTURE | PAN OR BUTTON HEAD SCREW WITH TORX (STAR) DRIVE RECESS, FULLY THREADED OR PARTIALLY THREADED METRIC SHANK. |
| KIND | INTERNAL / EXTERNAL |